Fireye 70D10 Flame Safeguard Chassis

Solid State Burner Management Series D

Type 70D40

| D-SERIES SOLID STATE BURNER MANAGEMENT CONTROLS | Bulletins D-30 D-1020 D-4041 |
| --- | --- |

Fireye Series D10, D20, and D30 burner management controls provide ignition and main flame failure protection for automatically ignited oil and gas burners. In conjunction with the operating controls and safety interlock devices, it automatically programs the burner/blower motor ignition, main fuel valves(s) and firing rate motor (Models 70D10 and 70D20 only). The control cycles automatically when the operating control closes and following a power interruption, but must be manually reset following a safety shutdown. Plug in programmers allow selection of purge and ignition timing. The D-Series controls utilize LED indicator lights to display the operating status of the control. Models 70D10 and 70D20 control the firing rate motor during light off. The Fireye 70D30 model does not have a firing rate motor circuit. Interchangeable programmer and flame amplifier modules allow complete versatility in selecting control functions. The D-Series controls can be used with ultraviolet, autocheck infrared, photocell, flame rod, or repetitive self-check ultraviolet flame scanners by selecting the proper flame amplifier.A complete control consists of: 70D10, 20 or 30 chassis, Programmer, Amplifier, Wiring Base, and a Scanner.The Fireye Series D40-41 Burner Management Controls provide ignition and flame failure protection for manually started oil or gas burners.

7lb