Raylink WISP AP Controller

# Radio Specifications

Frequency Range	2400 to 2.4835 GHZ

Operation Mode	CSMA/CA Carrier Sense Multiple Access/Collision Avoidance

Data Rate		2 Mbps

Channel Bandwidth	1 Mbps

Output Power	20 dB at antenna port

Sensitivity		1 Mbps –86dBm at antenna port, BER 1E10-6

2 Mbps –76dBm at antenna port, BER 1E10-6

Radio Type		FHSS (Frequency Hopping Spread Spectrum)

Bridging		Transparent Bridging

Layer2, Store and Forward

Store up to 2048 MAC addresses in bridge table

Broadcast Storm Protection

Packet Filtering at MAC level

Bandwidth Management, MAC access Authorization (Firewall)

128 Shaping Queues

Pre defined queue speed limit (from 32Kbps to 4096 Kbps)

Custom definable speed Limit

Queue assignment

>IP address (up to 128 user definable record of IP hosts and or Network address

>MAC address (up to 2048 MAC address)

Status

>Active - running based on settings

>Inactive – keep setting (queue in bypass made)

>Discard – setting in active mode (data will be blocked/discarded by user)

Quality of Service (Physical port Level)

3 data flow priority queues

Transmitted packets data prioritization

# Device Time Synchronization

Auto Synchronization

Modulation		Multilevel GFSK

# Configuration and Management

# Local Management

Monitor via port, using-based configuration utility

# Remote Management

Windows-based Configuration Utility

Remote Management Access

Management Access list filtering

From Ethernet or from the wireless link

# Software upgrade

TFTP upload via windows-based configuration utility

# SECURITY

# Security

RC4 Authentication, IP Address Access list, and Protocol filtering

# INTERFACE

Interface		 N Jack (female) connecter

Ethernet		10BaseT,RJ-45

Monitor Port  	9-pin Serial port

Power 			DC power port to external power supply

ADAPTER/PHYSICAL SPECIFICATIONS

Network Topologies	Infrastructure Access Point

Infrastructure Client

Ad- Hoc

MAC Protocol	IEEE 802.11 (CSMA/CA

Temperature 		0°C to 55°C (operation)

-20°C to +60°C(storage)

# COMPATIBILITY

OS Support 		Windows 98/ME/NT/2000/XP